DC No-Stic (No Stiction) v1.0

Copyright (c) 1991 Double Click Software

Program by Michael B. Vederman and Keith Gerdes

DC No-Stic (No Stiction) v1.0 is a FREEWARE program.  A FREEWARE program is

one which you may freely distribute, but you cannot charge for it or alter

the program or contents of the documentation in any manner.  Double Click

Software retains all copyright exclusive rights except exclusive distribution.

==============================================================================

DC No-Stic is designed to prevent hard disk stiction.

Stiction is when your hard disk head sticks in one place.  (big trouble)

Seagate drives suffered from this phenomenon.

Recently, some hard disk manufacturers began putting in logic in their

controller ROMs to prevent stiction from occuring.  Basically, they move the

hard disk heads around after a length of disk inactivity.

DC No-Stic will move the hard disk heads around after 5 minutes of hard disk

inactivity.

==============================================================================

=====================

Installing DC No-Stic

=====================

>To install DC No-Stic:

-----------------------

1) Put DCNOSTIC.PRG in your AUTO folder.

2) Reboot your computer.  DC No-Stic will display a message when it installs.

NOTE: If DC No-Stic is unable to install itself, it will print 'Not Installed'

This happens if no hard disk is hooked up, or not enough vbl slots exist.

- or -

1) Run DCNOSTIC.PRG.  You will be presented with an introdutory alert.

2) Click on INSTALL.

3) DC No-Stic will display a message.

================

Using DC No-Stic

================

If you are using a Syquest removable hard disk, you will want to relog all

partitions after changing cartridges.

>To relog all hard disk partitions:

-----------------------------------

1) Press the <ALTERNATE>+<CONTROL>+<LEFT SHIFT>+<F10> keys.

After 5 minutes of hard disk inactivity, instead of moving the heads

around, DC No-Stic will actually log in the partitions, then perform

the scheduled head moving next time.

NOTE: When the head movement occurs, mouse and keyboard input will halt.

This is to prevent the 'phantom typist' from occuring when the head

positioning occurs.  You won't notice anything but a momentary pause.

============

HOW IT WORKS

============

DC No-Stic will monitor hard disk activity.  Once all hard disk activity has

stopped, DC No-Stic will start counting.  When 5 minutes has elapsed,

DC No-Stic will seek to the beginning and end of each partition on all hard

disks connected.

NOTE: It will not be able to tell a RAMdisk from a hard disk.  (No big deal)

==============================================================================

This program is supplied in the belief that it operates as specified,

neither Double Click Software (the company) nor the authors shall not be liable

in any circumstance whatsoever for any direct or indirect loss or damage to

property incurred or suffered by the customer or any other person as a result

of any fault or defect in the goods or services supplied by the company

and in no circumstances shall the company be liable against consequential

damage or loss of profits (whether or not the possibility thereof was

separately advised to it or reasonably forseeable) arising from the use or

performance of such goods or services.

==============================================================================